*****************************************************************************

**

**          Mobile rOBot Simulation (MOBS)  - Documentation Version 2

**

**                     Horst Stolz, 7-Dec-1994

**                Time-stamp: <95/03/22 14:58:23 stolz>

**

*****************************************************************************

Table of Contents

-----------------

1. Introduction

2. Legal

3. Newest Version

4. Requirements

5. Installation

6. Recompilation

7. Overview

8. Users-Guide

8.1 Starting the Simulator

8.2 The User-Interface

8.3 Examples

8.4 Predefined objects/worlds and applications

8.5 Make own Robot-Applications

9. Command-line options

10. Bug-Reports and future

11. Author

Appendix

A. Robuter-Commands

B. Module-Splitting

C. File-Format

D. References

Keywords: simulation, robot, vehicle, ultrasonic-sensor, camera-view,

--------- geometric algorithm, gui, cross-talk, us-reflection

1. Introduction

***************

MOBS  ist  the abbrevation for "Mobile rOBot Simulator".  The simulator

emulate the behaviour of the mobile robot "RobuterII", which is a available

product  of  the  french  company  "Robosoft".   The  aim  is  to  build an

inexpensive   development-environment   for  writting  sensor-based  robot-

applications.

This  robot is a command-driven vehicle with up to 24 ultrasonic-sensors

at  the sides and cameras attached to the working-plate.  If you don't have

a  real  Robuter  or  at  least  the  manuals (-> appendix-References) this

program may be useless!

The  user  has the possibility to create and edit his own 3D-enviroment

by placing and changing simple objects or the predefined "Robuter" into the

simulation-world.   The  simulator  emulates  the  command-interface of the

robot  and computes the tof-values for the ultrasonic-sensors or generate a

image from the view of the camera.  The user can input the robuter-commands

by hand or write his own 'intelligent' applications and connect them via an

interface-library to the simulator.

2. Legal

********

The MOBS-package is public-domain and comes with no warranty.

3. Newest Version

*****************

The newest version of MOBS can be obtained via 'anonymous ftp' from:

ftp.informatik.uni-stuttgart.de

4. Requirements

***************

For the full functionality of the simulator, the following

packages must be installed on your computer:

1. XView

For the User-Interface.

At least the includes and the xview- and olgx-library!

2. DevGuide

The Interface-builder. For recompilation, the includes and

the guide- and guidexv-libraries must be available.

(set the GUIDEHOME-variable to the devguide-directory)

3. PVM-library

For communications between differenc processen.

4. OpenInventor (from SGI)

This is needed for the camera-view-generation.

5. Installation

***************

After unpacking you get the following directory-structure of the

simulator-package:

mobs--+--mof                     predefinded world- and object-files

|

+--iv                      Inventor-files for 3D-Displaying

|

+--src                     Source-Code

|

+--bin.$(HOSTTYPE)         Binaries

|

+--cmd                     Scripts (only necessary for compiling)

|

+--lib.$(HOSTTYPE)         interface-library and includes for

|                          connecting a robot-application

+--include                 to the simulator

|

+--example                 An example Robot-Application.

The simulator is divided into two executable programs: 'mobs' and the

render-server 'ivd_server' which generate a image from the

camera-view. It's not necessary that this programs are compiled for

the same machine! Their should be also some helpful example-programs

in your bin-directory: rob_get_frame, rob_tip, rob_us_drive and

rob_camera_example. If you missing one of the executables, please look

into the Recompilation-Section for compiling them.

Now set the Env-variable MOBS_HOME and add $MOBS_HOME/bin.$(HOSTTYPE)

and $MOBS_HOME/cmd to your path-variable.

Example commands to add to your '.cshrc'-startup-script if you have

unpack the packages in your home-directory.

...........................................................................

# .cshrc-addition

if (-d ~/mobs) then

setenv MOBS_HOME ~/mobs

set path = ( $path $MOBS_HOME/bin.$HOSTTYPE )

endif

...........................................................................

NOTE: The ivd_server use the MOBS_HOME-variable for referencing the

files in the $MOBS_HOME/iv-directory.

Now add at least the bin-directory in which the ivd_server-program is

located to your .pvm-hosts-file.

Example '.pvm-hosts'-file if you have a SGI namend 'munch' and a SUN4 named

'salvador':

...........................................................................

# first entry: hostname

# second entry: place of executables

#

# second entry: place of executables

#

# pvm-examples

salvador ep=/home/stolz/mobs/bin.sun4

munch ep=/home/stolz/mobs/bin.iris4d

#

## or more elegant:

##* ep=$MOBS_HOME/bin.$HOSTTYPE

##salvador

##munch

...........................................................................

6. Recompilation

****************

For recompiling the whole simulator, be sure that your system fullfill

all requirements.

Enter  the src-Directory and check the 'Defines.$(HOSTTYPE)'-file if it

fits to your machine. So choice your ANSI-C compiler and the change the

Include-Paths.

Now  use  gmake  (GNU-Make)  for  compiling  the  executables  and some

examples with the command 'gmake'

After  an success, use 'gmake install' to copy the new executables into

the ../bin.$(HOSTTYPE)-directory which is normally in the mobs-directory!

Now  you  can  delete  the object-files and other obsolent backup-files

with 'gmake clean'.

Thats it!

HINTS:

It's possible to use the simulator without the PVM-Library. But than

you can't connect a extern robot-application to the simulator. Also

The rendering of the camera-view is not available! But It is at least

possible to enter Robuter-Commands by hand.

To do this add the -DNO_PVM option in the CFLAGS=.. line of the

Defines.$(HOSTTYPE) file.

Than compile the executables and examples with:

'gmake mobs'  and 'gmake examples'

And don't forget 'gmake install' and 'gmake clean'.

If you don't have the OpenInventor-Library (from SGI) but have PVM

installed, you can also make with the above commands a running

simulator.

7. Overview

***********

The Simulator use a hierarchic object-model for repraesenting obstacles

and robots in an 3D-world. Each object has the following attributes:

- name, position, orientation

- function

- shape

A  function  of  an  object  can  be:   obstacle,  us-sensor, camera or

controller.   The  controller is the most important function and is able to

use the belonging sensors and cameras or set the position/orientation of an

object (actuator).  The shape of an object is a rectangle, triangle, cuboid

or group.  A RobuterII is therefor a assembled of many objects. Predefined

Robuters can be loaded and simple fundamental objects like "wall", "chair",

etc. are available to build a 3D-world.

All  computations  for  collision-detection, tof (time of flight) of an

ultrasonic  signal  are  done  in  a  3d-geometric-model.   The fundamental

element of this model is the triangle.  So all object-shapes are reduces to

triangles.

The   user   should  also  notice  the  distinction  in  the  geometric

visualisation  of  an  shape.  The shape of an object is for the 2D-display

normally  visualised by his edges.  But this can be exchanged to a serie of

polylines.  This is for example done for the default Robuter to get a nicer

appearance.  For the rendering in 3D, it's also possible to give a optional

visualization file.

For  simulation the ultrasonic-sensor, two choices exist.  Normally the

perfect-distance  mode  is  active.   The  time-of-flight  of the signal is

computed   by   the  distance  from  the  sensor  to  the  next  object  in

sensor-direction.

You   get  a  more  realistic  result  with  the  geometric-ultrasonic-

scan-mode.  In this mode, the ultrasonic wave is simulated by replacing the

sensor-area   with   triangles.   This  represent  a  section  through  the

ultrasonic-field.   The  triangles are than projected onto the triangles of

the  obstacles.   The  common area will be reflected, the rest-area will be

used  for  further  testing with the next obstacle.  If the signal also hit

the   inner   or   other   edge  of  a  cuboid,  a  special  wide  spreaded

edge-reflection  will be constructed.  The activation of the sensor depends

now  on  the  relation  between  received  and  transmitted intensity.  The

receiving  intensity  is  calculated  by the length of the signal path, the

acoustic   absorption   of   the   air,   the  reflection-factors  and  the

hitting-angle of the objects.  The tof-result the air-speed is necessary.

Please  note  that  this  scan-mode  takes  a while for computation.  So be

CAREFULL!   Set  the  maximum-reflection-count  which defines the number of

considered  multi-reflection as low as necessary!  The algorithm is able to

consider  reflections  on  walls  and  edges and the cross-talk between two

different sensors.

8. Users-Guide

**************

In this section, you find a helpful hints how to start the simulator, a

over-view   of   the   user-interface  and  a  course  to  build  your  own

robot-application-program.

8.1. Starting the Simulator

---------------------------

Make sure that you have set the MOBS_HOME-Variable to the mobs-packages,

that  the  bin-directory  is  in you path and you have created a pvm-config

file.

Now start PVM with your hostfile as parameter. Make sure that you have

halted a old pvm-process and type this in the terminal-window of your

Display-machine for which you have and 'ivd_executable' (normally a SGI).

Start the simulator with 'mobs' if you have the 'ivd_server' for the

same machine or 'mobs -renderhost <hostname>' if the server is only

available for another architecture.

EXAMPLE:  You have the 'mobs'-binary for SUN4 (name salvador) and the

'ivd_server' only for IRIS4D (name munch).  Start the simulator on salvador

with 'mobs -renderhost munch' if you are sitting on the munch-display.

PROBLEMS:

1) You get a warning that pvm ist not running.

->limitations: You can't connect extern application to the simulator!

Also the server-process for the camera-view-generation is failing!

Solution: Start pvm!

2) You get the warning that the 'ivd_server'-process can't be started.

->limitations: You have no camera-control-window and a camera-request

results in a empty-frame.

Solution: Add the path to the 'ivd_server'-binary to the pvm-hosts-file.

8.2. The user-interface

-----------------------

After  starting  the  simulator  the  working-windows  to  control  the

simulator and the camera-control-window which displays a 3D-view comes up.

working-window

~~~~~~~~~~~~~~

On  the top of the window, you found menus for loading/saving/inserting

files  into  the  simulator,  a  About-window  where you can also found the

pvm-task-id  of  the  simulator.  Also menu-items for an object or an robot

which pop up a control-window.  Some parameters which influence the picking

-behavoir of the editor, or the simulator can also be found.

On the right side of the window, a great working-area is present.  This

is    the    2D-display    of    the   simulation-world   from   above   in

parallel-projection.  Use the right mouse-button for the edit-menue in this

2D-display-area.   The edit menu let you insert a varity of simple objects,

group  them together or to zoom in/out.  The scale-factor can also set with

the  scale-slider  at  the  left  side.   The middle mouse-button moves the

center  of  the 2D-display to the mouse-position.  The left mouse-button is

for selecting, moving or rotating an object.

On  the  left  side of the working-window, you found buttons to control

the  Simulation  (Start, Stop, Step) and the stepsize (Frames/s).  With the

Reset-button  you  can reset all robot-command-interpeters and set the time

to zero.

Below  the  simulation-control  you found some switch which affects the

2D-display   and   the   simulation.    You  can  enable  the  display  the

object-names,  track  the motion of the robots or display the result of the

ultrasonic-scans.   With  the  triangle-view  switch the 2D-display objects

with their triangles.

To use the geometric-us-algorithms which was mentioned in the overview,

use   the   geometric-us-scan   switch.    Only   if  this  is  active  the

no-edge-reflection   switch  take  affect.   Please  don't  panic  if  than

computation  takes  a while!  Instead use the maximum-us-reflecion-count in

the env-configuration-window for fine-tuning.

camera-control-window

~~~~~~~~~~~~~~~~~~~~~

The camera-control-window comes only up if the 'ivd_server' is running.

It  displays  simulation-world  in color and 3D with simple rendering.  The

viewpoint  is  independent  from the viewpoint in the 2D-display.  But if a

robot-application  request  a  camera-frame,  the  3D-display switch to the

camera-position.

Note  that  the  3D-display  can  differ from the 2D-display cause it's

possible the set an own inventor-file for an object.

robot-control-window

~~~~~~~~~~~~~~~~~~~~

This  window shows you the position, angle and name of the robot.  Also

all  entered  inputs  (commands) and outputs are hold with their times in a

text-window.

You  can  enter  commands  by  hand  or  specify  your  own  program as

remote-control.  Type in your program-name (incl.  path) and start it

with the SPAWN-Button. You can kill the connected program with the

KILL-button. Be sure that your program lies in a path of your

pvm-hostfile, or type the explicit path to your program.

Use HALT to stop the robot immediatly and RESUME to continue from this

state. The button RESET let you reset

NOTE: Don't expect that something happens if you don't use the START- or

STEP-button of the simulation-control.

NOTE2: The template of the remote-control-string is:

[<hostname>@]<program> [<options>]

Use '*' as escape-character to insert the next char directly,

without splitting into seperated string.

object-control-window

~~~~~~~~~~~~~~~~~~~~~

The  object-window  let's  you change the position, orientation, shape,

color or 3D-visualization of an object. The changes take effect after

pressing the Apply-Button!

The  3D-visualizatio  n  file  is an Inventor-File (->References) which

should   be   given  with  full  path-name  or  relativ  to  the  directory

$MOBS_HOME/iv.  This file will only loaded from the 'ivd_server' and has no

effect to the simulation or the 2D-display.

editor/simulator/environment-configuration

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In  this  windows  you  can set the picking and moving-behaviour of the

mouse   and   simulation-relevant  parameters  like  the  sound-speed,  the

air-damping the max-us-reflecion-count and the collision-stepsize.

8.3. Examples

-------------

The are some example-worlds in the mof-directory.

* Drive through a room with activated camera

Load with the Menu-item File/Load the file 'mof/camera_drive.mof'. Zoom out

so that you can see the robot on the bottom/left-corner. Select the robot

an activate the robot-control-window. Now swich on the Remote-control and

enter the programm 'rob_camera_example'. Now start the simulation. The robot

drive through the room and request the camera-view. It saves the images

under the files frame*.pgm.

* Crosstalk

Load the file 'mof/crosstalk.mof'. You should see two robots with

one us-sensor on the front-side. Then activate the

object-names-display and click on one robot. Popup the

robot-control-window and enter the commands 'rnod c=*****0'. Do the same

also with the other robot. Now activate the us-sensor-results and

press Step-button of the simulation-control until you get a

result. (this should happen after 6 clicks)

Now clear the results by disabling and enabling the

us-results. Activate the geometric us-scan and simulate stepwise until

you get a results. You should see than the robotA get a fake-activation

of the front-sensor.

* Multi-reflection

Load the 'mof/multi_reflection.mof'-file and enter also 'rnod c=*****0'

to activate the us-sensor reading of the front-sensor. Use the

geometric-scan-mode to get the reflections of the signal on booth obstacles.

You can also play with the max-reflecion-count in the Env-Config window.

* robot-driving with ultrasonic-sensors

Load the 'mof/us_drive.mof'-file. Select the robot on the top-right and

start the remote-application 'rob_us_drive'. Please switch off the

geometric-scan-mode to avoid frustrating waiting-times.

8.4. Predefined objects/worlds and applications

-----------------------------------------------

Predefined objects for inclusion: (use File/Insert)

mof/robuter.mof               A Robuter with one camera and 24 us-sensors

mof/robuter_wo_camera.mof     A Robuter with 24 us-sensors installed

mof/robuter_1uss.mpf          A Robuter without camera and with 1 us-sensor

mof/porthos.mof               8 us-sensor-Robot namened 'porthos' with camera

mof/athos.mof		   12 us-sensor-Robot namened 'athos' with camera

mof/aramis.mof                12 us-sensor-Robot namened 'aramis' with camera

For making white driving-signs on the floor, use the following objects

mof/circle_r1500_a15.mof      A circle with radius 1.5m build with rectangles

15degree-steps

mof/circle_r1500_a30.mof      A circle with radius 1.5m build with rectangles

30degree-steps

mof/eight.mof                 A lying eight.

Whole worlds: (use (File/Load)

mof/robolab.mof               The Robuter-Labratory of the IPVR.

mof/camera_drive.mof          A room with some interior

mof/crosstalk.mof             Example to show crosstalk between two robots

mof/multi_reflection          Example to show reflexion on two obstacles

mof/us_drive.mof              Again the robo-lab with two robuters.

Executable applications in the bin-directories:

rob_get_frame              Request a camera-frame (use -h for help)

(can also save the frame into a file!)

rob_tip                    Let you enter commands by console-io.

rob_camera_example         Drive robot on predefined path through

example-world 'camera_drive.mof'

rob_us_drive               Simple program to drive 24-us-sensor-robot

with obstacle-avoidance for an 24-us-sensor-Robot!

8.5. Make your own Robot-Applications

-------------------------------------

To build your own application, the interface-library

'librob_mobs.a' is and the corresponding head-file 'rob.h' is

needed. They should be exist in the lib- and include-directory.

NOTE: You can also make the library with:

'cd $MOBS_HOME/src; gmake lib; gmake install; gmake cleanup'

A example robot-application can be found in the example-directory.

Use 'gmake' to compile the example.

9. Command-line-options

***********************

Options of the Simulator 'mobs':

Usage:  mobs [-ivd <tid-nr>] [-renderhost <hostname>]

[-renderdisplay <display>] [-renderfile <filename>] mobs-object-files ...

with:

-h                      help

-ivd <pvm-tid-nr>       connect to already running camera-view-server

-renderhost <hostname>  Specify the host-computer for the camera-server

(default is current host)

-renderfile <fname>     Root-renderfilename for the camer-view

(default is lights.iv)

-renderdisplay <dply>   Display-host (eg.: "roy:0.0")

10. Bug-Reports and further development

**************************************

If you found bugs or have a constructive idea send a e-mail to:

stolz@hermes.informatik.uni-stuttgart.de

When you don't get a answer within 7 days use the following e-mail or

snail-mail address of my project-leader:

braunl@hermes.informatik.uni-stuttgart.de

Dr. rer. nat. habil. Thomas Braeunl

Universitaet Stuttgart, Fakultaet Informatik

Institut fuer Parallele und Verteilte Hoechstleistungsrechner

Abteilung Bildverstehen

Breitwiesenstr. 20-22

D-70565 Stuttgart

(Germany)

But  the  further  development  depends  on  if  someone can be found which

mantains the software, adds new functionality or rewritte some ugly code.

11. Author

**********

Name:        Horst Ulrich Stolz

snail-mail:  Pfalzgrafenweg 22

71083 Herrenberg

Germany

e-mail:      stolz@hermes.informatik.uni-stuttgart.de

(no guarantee for an answer after 1.1.1995)

#### APPENDIX #############################################################

A. Robuter-Commands

*******************

The following list is only a overview over the implemented

Robuter-commands. For functional details, look into to robuter-manual.

BUMP (ON | OF[F])

RESET

-Basic Motion commands:

MOTN (ON | OF[F])

MOTN (FO[RWARD] | BA[CKWARD]) [D=<distance>] [V=<velocity>]

MOTN (LE[FT] | RI[GHT]) [A=<angle>] [V=<velocity>]

MOTN (KI[LL] | WA[IT] | ST[ATUS])

- Advanced Motion commands: (with motion-queuing)

MOTV (ON|OF[F])

MOTV (AD|LA) [V=<linear velocity>] | V+ | V-]

[W=<angular velocity>] | W+ | W-]

[D=<distance> | A=<angle> | T=<time> | IN]

[L=<linear acceleration> | L+]

[K=<angularacceleration> | K+]

MOTV KI ([L=<linear acceleration>|L+] [K=<angularacceleration>|K+])

MOTV RS

MOTV MS

MOTV NS

- Odometrie:

ODOM (ON|OF[F])

ODOM

ODOS C=<x>,<y>,<angle>

ODOF C=<dx>,<dy>,<dangle>

- Ultrasonic configuration and reading:

SSET S=<sensor desciptor> [N=<sensor#>] [A=<angle>] [D=<direction>]

RNOD C=<configuration>

RNOD R=(ON|OF[F])

READ C=<node>,<sensor>,<range>

READ S

- Commands only available in the simulator:

HALT             ... stops the robot and ignore all commands except 'resume'

RESUME           ... escape from the halt-mode.

Module splitting

****************

The following table gives an brief overview of the modules in the

src-directory and their place in the simulator.

graphical user interface:

------------------------

xvmobs_stubs.c                  user-interface-callbacks and main().

xvmobs_ui                       user-interface definition

displaying:

----------

app2d                           A Line-Definition for 2D-displaying

xdraw                           2D-Display-routines of the mob-world for X11

robot emulator:

-----------------

command                         entry point of the command-interpeter

coms_us                         emulation of RNOD, SSET and READ-command

com_motn                        the MOTN-command

com_motv                        the MOTV-command

com_scanner                     functions for command-scanning

mob                             the Robuter emulation

vehicle                         a acceleration-driven 2D-vehicle

mob_limits.h                    limits of the Robuter

interface-library:

------------------

rob                             the robot-library

mobs_interface                  the client to the pvm_server of MOBS

generic modules:

----------------

basic                           some fundamental macros and math-functions

darray                          dynamic array for constant sized elements

error                           holds error and warning-functions!

integration                     integration-module for classical runge-kutta

list                            single-linked list-datatype.

parray                          dynamic pointer-array

pdlist                          priority-list for double-piorities

plist                           module for a piority-list (long-priority)

rev.h                           holds version and project information

queue                           queue for constant sized elements (->stack)

scheduler                       'process'-scheduler for simulation-purposes

strerror                        for systems with undefined strerror().

text                            a abstract text-type to store text-infos.

vector                          2- and 3-vector defs and operators.

vector_geometry                 simple routines for vector-geometry problems

triangle-model:

---------------

clipping                        some clipping-functions

polygon                         triangulation of a polygon

space                           space-segmentized triangle-model

triangle                        fundamental triangle-routines

triangle_projection             compute rest and common faces of a projection

object-model:

-------------

us_sensor                       the ultrasonic-sensor-object

mwo                             module provides primitive objects

mwo_mob                         the robuter-object

mw_file                         file i/o

mw_object                       basic object functions

mob_world                       object-model for the simulated world

communication:

--------------

pvm_server                      command-connection MOBS<->robot application

ivd                             connection MOBS<->camera-view-generation

ivdp_obj.h                      some defs for the above module

camera-view-generation:

-----------------------

ivd_server.c                    camera-view-generation with Inventor-Lib

iv_draw                         module to display a object with Inventor

example code:

-------------

ivd_tip.c                       sends special commands to the ivd_server!

ivmobs.c                        iv-display of a mof-file

rob_camera_example.c            example-code for obtaining a camera-frame

rob_get_frame.c                 request/save a single camera-frame

rob_tip.c                       command-io via console

rob_us_sensor                   module for us-sensor reading

us_sensor_drive.c               example for driving a 24-sensor-robot

test code:

----------

conmobs.c                       for testing rob-commands via console

edge_reflexion_test.c

ivd_test.c

space_test.c

triangle_projection_test.c

triangle_test.c

rob_camera_test.c

roblib_test.c

iv_space_test.c

mobs_interface_test.c

mobs_pvm_test.c

mw_test_io.c

polygon_test.c

C. File-Format

*******************

The  simulator can store the contents of a whole simulation-world or single

objects  into a file.  Here is the definition of these "Mobs Object Format"

(MOF) in extended Backus-Naur-Form (EBNF):

# Remark to end of line!

# token-types:

# { } " keywords, <int>, <double>, <string>, <vector3>, <quaternion>

<MOF-file> = [<world>|<object>]

<world> =  world {

[dt_collision_test <double>]

[air_speed <double>]

[us_air_damping <double>]

[objects {

[<object>]+

}]

}

<object> = (group|triangle|rectangle|cuboid) {

[<obstacle>|<mob>|<sensor>|<camera>]

[name "<string>"]

[position <vector3>]

[orientation <quaternion>]

[us_reflexion_damping <double>]

[marked (true|false)]

[<appearance>]

[visualization "<string>"]

[actuator <nr>]

[<members>|					# group

edges <vector3> <vector3> <vector3>|	# triangle

xy_size <double> <double>			# rectangle

edges_length <vector3>]			# cuboid

}

<members> = members { [<object>]+ }

<appearance> = appearance {

nr_points <int>

[code_length <int>

points {

[vector]+

}

code [<int>]+]

}

<obstacle> = obstacle

<mob> = mob {

geometry <a:double> <b:double> <c:double> <d:double>

<r1:double> <r2:double>

}

<sensor> = sensor {

nr <int>

}

<camera> = camera {

nr <int>

angle <float>

focus_distance <float>

aspect_ratio <float>

}

<vector3> = <double> <double> <double>

<quaternion> = <double> <double> <double> <double>

keyword = (a..z|A..Z) [a..z|A..Z|_]+

D. References

*************

Inventor: Iris Inventor Programming Guide; Silicon Graphics, 1992

PVM: PVM 3 User's Guide and Reference Manual; Oak Ridge National

Laboratory, Engineering Physics and Mathematics Division, 1993

Robosoft:  Albatros User's Manual V5.3; Robosoft S.A., Neuilty-Plaisance,

France, April 1993

Robosoft: Local Aera Ultrasonic Network - User's Guide; Robosoft S.A.,

Neuilty-Plaisance, France, April 1993

Robosoft:  Robuter User's Manual V4.0; Robosoft S.A., Neuilty-Plaisance,

France, April 1993

Stolz, Horst:  Simulationssystem fuer autonome mobile Systeme mit

Schwerpunkt auf Ultraschallsensorik; Diplomarbeit Nr.  1170, Universitaet

Stuttgart, IPVR, Abt. Bildverstehen, November 1994